print name (first last):	______________________

course: ECET 337     						2025-11-18

lab section (day  time):	______________________

Basic Op Amps

+ 5V

Checks									Prelab			Performance

In lab   or  Late

_____   1.	Voltage Divider					_____(16%)		_____(16%      or     8%)

_____   2.	Noninverting Amplifier 			_____(19%)		_____(19%      or   10%))

# Lab Performance Scoresheet

Prelab initial submission (on time, with data, complete) …	(35%)			_________

All required signed Performance check offs	………………	(35%)			__________

Total	(out of 70%)		__________

Comments:

Prelab Activity

- If you need help with the following problems, look at the guide attached at the end of this lab exercise.

- On the Course’s Brightspace site, access the 01-Labs-Basic Op Amps -Prelab-Demonstration module.  Watch the videos.  Extract from those demonstrations the data for the following procedures.  Record those data in the tables provided below.  Calculations are not required as Prelab.  You will complete and demonstrate the calculations during the lab period.

Objectives

- Design a voltage divider and investigate the effect of load.

- Design a voltage follower and a noninverting amplifier to compensate for loading.

Approach and Results

- Voltage divider

- Build the schematic shown in Figure 1.  Do not connect the 5 VDC source to the divider, yet.

Figure 1  Voltage divider schematic

- Measure the value of the 22 k resistor and the end-to-end resistance of the potentiometer. Record their value in Table 1.  Record all stable digits.

- Connect the 5V source to your voltage divider, and record the input voltage, Ein measured in Table 1.  Record all stable digits.

Table 1  Voltage divider components

|  | Prelab | Actual (k) |
| --- | --- | --- |
| R1 (k) |  |  |
| R2 (k) |  |  |
| Ein measured (V) |  |  |
| Vnode A measured (V) |  |  |

- Calculate and measure the DC voltage at node A (with respect to common), using the actual (measured) component values.  Put the measured value in Table 1.  Record all stable digits.

Vnode A calculate = ___________

- Rework your circuit and measurements until the error is within +5%.

- Measure the voltage at node B (the wiper of the potentiometer) with respect to common. Adjust the potentiometer to produce VB DC = 1.00 VDC.

- Add a 3.3 k resistor between the wiper and common, as shown in Figure 2.

Figure 2  Loaded voltage divider

- Measure the DC voltage across this 3.3 k load.

Table 2  Loaded voltage divider

|  | Prelab (V) | Actual (V) |
| --- | --- | --- |
| Vload measured |  |  |

- Why has the voltage divider “loaded down”?  Write your answer below.

- Calculate the voltage you expect across the load, taking into consideration the fact that the load resistor is in parallel with the lower resistance of the voltage divider.  Include your calculations below.

Figure 3  Unloaded and loaded voltage divider calculation schematics

Demonstrate the circuit’s performance, the calculations, and the data to your lab instructor.

- Disconnect R1, the 22 k resistor, from the 5 V source.

- Remove the load resistor from the output of the voltage divider.

- Voltage Follower

- Add the voltage follower to your circuit, as shown in Figure 4.  Use the very top bus for the +5 V supply and the very bottom bus for the – 5V bus.  Make the inner-bottom bus the common bus.

Notice that there is a short (the dotted line) connected between the op amp’s noninverting input, pin 3 and common.

Also, R3, the 3.3 k load resistor has been moved so that it can be connected to the op amp’s output, pin 6.  However, do not connect it, yet.

Figure 4  Voltage follower schematic

- Remove the connection between pin 3 of the op amp and the wiper.  But, leave the jumper between pin 3 and common.  Pin 3 is connected to common but not to the potentiometer’s wiper.

- Turn the power supply on.

- Measure the output of the voltage follower (op amp, pin 6) with respect to circuit common using the multimeter.  If that voltage is greater than +100 mVDC, rework your circuit.  Continue only when the op amp’s output voltage is appropriately small.

- Turn the power supply off, remove the short-to-ground at the op amp’s input, then connect the input of the op amp (pin 3) to the unloaded output of the voltage divider (R2).

- Turn the power supply on.

- Measure the voltage at the wiper of the potentiometer and the output from the op amp, the voltages indicated in the first row of Table 3.  Continue only when there is less than 5% error.

Table 3  Voltage divider and voltage follower results

|  | Prelab (VDC) Using actual component values | Measured (VDC) | Error (%) With respect to Expected |
| --- | --- | --- | --- |
| Unloaded (2f) |  |  |  |
| Loaded (2h) |  |  |  |

- Connect the 3.3 k resistor between the output of the op amp , pin 6, and circuit common.

- Measure the voltages indicated in the second row of Table 3.  Continue only when there is less than 5% error.

- Explain why the simple voltage divider was loaded by the 3.3 k resistor, but that the voltage from the voltage follower was correct.  Write your response below.

- Turn the power supply off.

- Noninverting Op Amp Amplifier

- Alter the voltage follower into the noninverting amplifier shown in Figure 5

Figure 5  Noninverting amplifier

- Measure the resistors you added.  Record all stable digits.

Table 4  Amplifier components

|  | Prelab (k) | Actual (k) |
| --- | --- | --- |
| Ri |  |  |
| Rf |  |  |

- Calculate the theoretical output voltage using the actual values of Ri and Rf, with an input of 1.00 VDC.  Record that value in the proper cell of Table 5.  See the attached tutorial if you don’t remember the gain equation for a noninverting op amp amplifier.

- Turn the power supply on.

- Set the input voltage (node B with respect to common) to 1.00 VDC.

- Measure the voltage at the output and record its value in Table 5.  Continue only when there is less than 5% error.

Table 5  Noninverting amplifier results

| Prelab (VDC) | Expected (VDC) Using actual component values | Measured (VDC) | Error (%) With respect to Expected |
| --- | --- | --- | --- |

Demonstrate the circuit’s performance and the voltmeter display and the data to your lab instructor.

- Turn the power supply off.

Analysis & Conclusions

Write two paragraphs.  In the first, explain why the 3.3 k resistor loaded the voltage divider, but placing the voltage follower between the voltage divider and the 3.3 k load prevented the loading.  In the second, discuss the errors you observed; their magnitudes, the causes, and possible solutions.

Fundamental Analog Principles

Voltage Divider

Ohm’s Law defines the relationship among the voltage (pressure), current (flow), and resistance (friction) in an electrical circuit.

V=I×R          I=VR       R=VI

The units for V is V, volts;      I is A,amperes  or amps    R is , ohms

Placing two resistors end-to-end, in series, causes the total opposition to add, shown in Figure 1.

Figure 1 Resistors in series to form a voltage divider

Rtotal=R1+R2

The voltage divider, shown in Figure 1, is used to reduce the input voltage ein to a lower (more usable) level, vout.  The input voltage ein is applied across Rtotal, producing a current Itotal.

Itotal=einRtotal

This current flows through R1, then R2.  So , IR2= Itotal

vout=vR2=IR2×R2

vout=Itotal×R2=ein×R2Rtotal

vout=ein×R2R1+R2

or

voutein=R2R1+R2

Op Amp Voltage Follower

One of the most common linear op amp circuits (one that uses negative feedback) is the voltage follower.  Before beginning the analysis of this circuit, however, you must recall two facts about the op amp itself.  These are illustrated in Figure 2.

First, since the input impedance is very high, the signal current that flows into either input terminal 
(- or +) is negligibly small.

Figure 2  Fundamental op amp characteristics

Second, the open loop gain, vout/vin, is extremely large.  This means that for any reasonably sized output (any output in the linear region, not VSAT), vin must be negligibly small.

The two inputs leads are at virtually the same potential.  You certainly will never be able to measure any significant potential difference between the two inputs during linear operation.

Schematics for a voltage follower is given in Figure 3.

- Simple                                      b)  Connected to the voltage divider

Figure 3  Voltage follower

There is no difference between the two inputs.  This means that since the noninverting input is at the source potential, so is the inverting input.  However, the inverting input is tied directly to the output.  The output voltage equals the voltage at the noninverting input, which is the input voltage.  So the 
output voltage equals the input voltage, giving a gain of 1.

Remember, that no significant current flows into the input of the op amp.  So, this circuit will draw no current from ein.  More importantly, if a voltage divider (Figure 1) with R1 and R2 were connected between ein and the op amp, none of IR2 would flow into the op amp.  The voltage across R2 would not change.  It is said that the op amp (Figure 3) does not load down the voltage divider (Figure 1).

Noninverting Amplifier

The voltage follower uses 100% negative feedback.  But, as a result, it produced an output voltage no larger than its input.  To allow a larger output, you must reduce the amount of negative feedback.  This is done by the voltage divider made of Rf and Ri in Figure 4.

Figure 4  Noninverting amplifier

The gain, Av is

You can derive the voltage gain by recalling that there is virtually no difference between the two inputs of the op amp (during linear operation with negative feedback).

Since no significant current flows into the inputs of an op amp, the current through Ri is also the current through Rf.

The voltage dropped by this current across Rf is

The output voltage is the voltage across Ri plus the voltage across Rf.

Since the gain is the output voltage divided by the input voltage,

The Effect of Loading and Buffering

Placing a load (a resistor or something else to use the output voltage) across the output of a voltage divider lowers that voltage.  The act of taking energy out of a circuit lowers the energy available.  That is loading.  This is shown in Figure 5a.

- Loaded voltage divider                                   b)  Op amp buffer

Figure 5  Loading and buffering a voltage divider

Without the load resistor, all of the current that flows through R1 then flows through R2, producing vout.  However, connecting Rload in parallel with R2 means that some of the current from R1 now is diverted to Rload.  This lowers the current available to R2 and that lowers vout.

However, placing a voltage follower (buffer) between the output of the voltage follower and the load prevents the loading.  In Figure 5b, no current flows into the input of the op amp.  So, all of the current from R1 flows through R2, just as it did without the load connected.  The voltage follower passes vout to its output, where Rload is connected.  The current for Rload now comes from the power supplies, through the op amp’s output pin.

Example

- Given R1 = 5 k, R2 = 10 k, ein = 3 V, calculate vout with no load

vout=3 V×10 kΩ5 kΩ+10 kΩ=2 V

- Calculate the effect of connecting Rload = 7 k.

Remember parallel circuits?  Connecting Rload in parallel with R2 produces a lower resistance.

Rparallel=11R2+1Rload=1110 kΩ+17 kΩ=4.1 kΩ

vout=3 V×4.11 kΩ5 kΩ+4.11 kΩ=1.35 V

This is a 35% reduction (loading down) of vout.  Using the voltage follower of Figure 5b will prevent this error.