ECET LABORATORY SAFETY RULES

Students working in any ECET laboratory agree that they are aware of and will abide by all applicable safety policies.  By working in any ECET lab a student also acknowledges that there may be inherent safety risks associated with such work, and continues to work in the lab at his/her own risk.

- In the event of an emergency, shut down the power to the experiment.  If necessary, use the lab panic button and notify your instructor.

- Students shall ask permission of the instructor/supervisor before using an open lab space.

- Students shall wear safety glasses at all times unless exempted by the lab instructor.

- Food, drink, or tobacco products are not permitted in the lab.

- Coats, book-bags, backpacks, and the like shall not be placed on equipment.

- A student alone in any ECET lab may only do keyboarding.  An open parts kit, parts and/or tools on the bench/table, cables attached to micros or PCs, etc., is as a violation of this rule.

- Voltages equal to or greater than 50V nominal shall be insulated to prevent accidental contact.

- All components and circuitry operating at or above 50V must be covered with a blast shield

- All circuits shall be de-energized when removing, repositioning or installing components or wiring.

- All circuits above 24V shall be de-energized when moving test leads.

- Conductive jewelry such as rings, watches, bracelets, necklaces, and the like shall be removed while working in the lab.

- Troubleshooting and testing for circuits operating below 24V shall be done using one hand.

- Students must report to the instructor/lab supervisor any defects or deficiencies in equipment at the lab station at which you are working, including test leads and power leads, especially soldering iron leads.

- Students shall be alert for any potential problems and always think about and plan what must be done before doing it.

- Horseplay will not be tolerated and will result in dismissal from the lab.

- Upon leaving a lab station stow stools/chairs under the bench, clear all trash to the appropriate receptacle, return all equipment to the standard setup for that lab, and finally wipe it down with a disinfecting sheet.

Any violation of these policies, as well as any other action that puts the student or anyone else at risk of bodily injury, will result in following:

- Your name will be taken and you will be instructed to leave.

- A second violation of this policy will result in your privileges to use any ECET open lab being revoked.